# Experiment Title

Generation of White Noise and Estimation of Signal-to-Noise Ratio (SNR) and Noise Figure of a System

## Aim of the Experiment

To generate white Gaussian noise, analyze it in time and frequency domain, and calculate the Signal-to-Noise Ratio (SNR) and Noise Figure (NF) of a given communication system.

## Apparatus / Software Used

- MATLAB / GNU Octave

- PC with minimum 4 GB RAM

## Theory

### White Noise

White noise is a random signal having constant power spectral density over all frequencies. In communication systems, it is usually modeled as Additive White Gaussian Noise (AWGN).

### Signal-to-Noise Ratio (SNR)

SNR is the ratio of signal power to noise power.

SNR = Psignal\Pnoise

​​In decibels:

SNR(dB) = 10 log10 (Psignal\Pnoise)

### Noise Figure (NF)

Noise Figure measures the degradation of SNR due to a system.

NF = SNRin \SNRout

​ In decibels:

NF(dB) = SNRin (dB)  −  SNRout (dB)

## Block Diagram

Signal Source ──► Adder ──► System (Gain + Noise) ──► Output

▲

White Noise

## Python Program (NumPy Based)

import numpy as np

# Parameters

fs = 10000           # Sampling frequency

N = 10000            # Number of samples

signal_amplitude = 1 # Signal amplitude

noise_variance = 0.1 # Noise power

# Generate signal (sine wave)

t = np.arange(N) / fs

signal = signal_amplitude * np.sin(2 * np.pi * 1000 * t)

# Generate white Gaussian noise

noise = np.sqrt(noise_variance) * np.random.randn(N)

# Input signal + noise

input_signal = signal + noise

# System (gain + added noise)

system_gain = 2

added_noise_variance = 0.2

system_noise = np.sqrt(added_noise_variance) * np.random.randn(N)

output_signal = system_gain * input_signal + system_noise

# Power calculations

signal_power = np.mean(signal ** 2)

noise_power = np.mean(noise ** 2)

output_noise_power = np.mean(system_noise ** 2)

# SNR calculations

snr_in = 10 * np.log10(signal_power / noise_power)

snr_out = 10 * np.log10((system_gain**2 * signal_power) / output_noise_power)

# Noise Figure

noise_figure = snr_in - snr_out

# Display results

print(f"Input SNR (dB): {snr_in:.2f}")

print(f"Output SNR (dB): {snr_out:.2f}")

print(f"Noise Figure (dB): {noise_figure:.2f}")

## MATLAB Program

clc;

clear;

close all;

% Parameters

fs = 10000;             % Sampling frequency

N = 10000;              % Number of samples

A = 1;                  % Signal amplitude

f = 1000;               % Signal frequency

noise_var = 0.1;        % Noise variance

% Time vector

t = (0:N-1)/fs;

% Generate sine wave signal

signal = A * sin(2*pi*f*t);

% Generate white Gaussian noise

noise = sqrt(noise_var) * randn(1, N);

% Input signal

input_signal = signal + noise;

% System parameters

gain = 2;

added_noise_var = 0.2;

system_noise = sqrt(added_noise_var) * randn(1, N);

% Output signal

output_signal = gain * input_signal + system_noise;

% Power calculations

signal_power = mean(signal.^2);

noise_power = mean(noise.^2);

output_noise_power = mean(system_noise.^2);

% SNR calculations

snr_in = 10 * log10(signal_power / noise_power);

snr_out = 10 * log10((gain^2 * signal_power) / output_noise_power);

% Noise Figure

noise_figure = snr_in - snr_out;

% Display results

fprintf('Input SNR (dB): %.2f\n', snr_in);

fprintf('Output SNR (dB): %.2f\n', snr_out);

fprintf('Noise Figure (dB): %.2f\n', noise_figure);

%% ----------- Time Domain Plots ----------------

figure;

subplot(3,1,1);

plot(t(1:1000), signal(1:1000));

title('Original Signal (Time Domain)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t(1:1000), noise(1:1000));

title('White Gaussian Noise (Time Domain)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,3);

plot(t(1:1000), input_signal(1:1000));

title('Signal + Noise (Time Domain)');

xlabel('Time (s)');

ylabel('Amplitude');

%% ----------- Frequency Domain Plots ----------------

f_axis = (-N/2:N/2-1)*(fs/N);

SIGNAL_FFT = fftshift(abs(fft(signal)));

NOISE_FFT = fftshift(abs(fft(noise)));

INPUT_FFT = fftshift(abs(fft(input_signal)));

figure;

subplot(3,1,1);

plot(f_axis, SIGNAL_FFT);

title('Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

subplot(3,1,2);

plot(f_axis, NOISE_FFT);

title('Noise Spectrum (White Noise)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

subplot(3,1,3);

plot(f_axis, INPUT_FFT);

title('Signal + Noise Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

## Sample Output

Input SNR (dB): 9.95

Output SNR (dB): 6.82

Noise Figure (dB): 3.13

## Observations

- The white noise waveform exhibits random amplitude variations in the time domain, indicating the absence of any periodicity.

- The frequency spectrum of the noise is nearly flat over the observed bandwidth, confirming the characteristics of white noise.

- The output SNR is lower than the input SNR, indicating degradation due to internally generated system noise.

Top of Form

## Result

White Gaussian noise was successfully generated using a random Gaussian process.
The Signal-to-Noise Ratio (SNR) at the input and output of the system was calculated, and the Noise Figure (NF) was evaluated.
The obtained results verify that the system introduces additional noise, thereby reducing the output SNR.

## Conclusion

This experiment demonstrates the generation and analysis of White Gaussian Noise, a fundamental noise model in communication systems. The system performance was evaluated using Signal-to-Noise Ratio (SNR) and Noise Figure (NF). The observed reduction in output SNR confirms the effect of internally generated noise,

- White noise is generated using Gaussian random variables

- SNR is calculated using signal and noise power

- Noise Figure quantifies degradation in SNR caused by the system

# Numerical Problems

### Problem 1

Input SNR = 30 dB
Output SNR = 22 dB
Find Noise Figure.

Solution:

NF = SNRin – SNRout = 30 – 22 = 8 dB

### Problem 2

Signal power = 20 mW
Noise power = 2 mW
Calculate SNR in dB.

Solution:

SNR = 10 log10 (20/2) = 10 log10 (10) = 10 dB

### Problem 3

A receiver has NF = 6 dB and input SNR = 28 dB.
Find output SNR.

Solution:

SNRout = SNRin – NF = 28 – 6 = 22 dB

### Problem 4

Noise power spectral density = 10−9 W/Hz
Bandwidth = 1 MHz
Find noise power.

Solution:

Pn=N0 *  B = 10−9 × 106 = 10−3 W

### Problem 5

A system has input SNR = 25 dB and output SNR = 18 dB. Find the Noise Figure.

Given:
Input SNR = 25 dB
Output SNR = 18 dB

Formula:

Noise Figure (NF) = SNRin − SNRout

​ Solution:

NF = 25 – 18 = 7 dB

### Problem 6

If signal power is 10 mW and noise power is 1 mW, calculate SNR in dB.

Given:
Signal power Ps = 10 mW
Noise power Pn = 1 mW

Formula:

SNR(dB) = 10 log10 (PsPn)

Solution:

SNR = 10 log10(10) = 10 × 1 = 10 dB

### Problem 7

A receiver has NF = 5 dB. Find SNRout if SNRin = 30 dB.

Given:
Noise Figure NF = 5 dB
Input SNR = 30 dB

Formula:

NF = SNRin − SNRout​

Solution:

SNRout = 30 – 5 = 25 dB​

### Group B_ Experiment No. 1

### Oral Questions

### Q1. Why do we use Noise Figure instead of noise power?

Ans: Noise Figure provides a normalized measure of system performance independent of signal level.

### Q2. What happens to Noise Figure when stages are cascaded?

Ans: Overall Noise Figure increases and depends heavily on the first stage (Friis formula).

### Q3. Why is white noise important in communication systems?

Ans: It accurately models random disturbances affecting signal transmission.

### Q4. What is the effect of bandwidth on noise?

Ans: Noise power increases linearly with bandwidth.

### Q5. What is thermal noise?

Ans: Noise generated due to random motion of electrons in conductors.

### Q6. How can Noise Figure be improved?

Ans: By using low-noise amplifiers and minimizing losses in the receiver front end.

### Q7. Difference between white noise and colored noise

Ans: White Noise:
Has constant power spectral density over all frequencies.

Colored Noise:
Has non-uniform power spectral density and varies with frequency.

### Q8. Why is Gaussian noise preferred in system modeling?

Ans: Gaussian noise is preferred because the combined effect of many independent noise sources follows a Gaussian distribution according to the Central Limit Theorem.

### Q9. How does bandwidth affect noise power?

Ans: Noise power increases linearly with bandwidth.

Pn=N0B

### Q10. Define noise temperature

Ans: Noise temperature is the equivalent temperature that represents the amount of noise power generated by a system.

### Q11. What is the effect of cascading stages on Noise Figure?

Ans: The overall Noise Figure of cascaded stages is dominated by the first stage, as given by the Friis formula.

### Q12. Define white noise.

Ans: White noise is a random signal having constant power spectral density over all frequencies.

### Q13. Why is noise assumed Gaussian?

Ans: Due to the Central Limit Theorem, the sum of many independent noise sources follows Gaussian distribution.

### Q14. Define SNR.

Ans: SNR is the ratio of signal power to noise power.

### Q15. Define Noise Figure.

Ans: Noise Figure is the measure of degradation of SNR caused by a system.

### Q16. Unit of Noise Figure?

Ans: Decibel (dB)

### Q17. Why is output SNR less than input SNR?

Ans: Because the system introduces additional internal noise.

### Q18. What does a low Noise Figure indicate?

Ans: Better receiver performance and higher sensitivity.

### Q19. Why is noise spectrum flat?

Ans: White noise has equal power at all frequencies.

### Q20. Relation between SNR and Noise Figure (dB)?

Ans: NF = SNRin − SNRout

### Q21. Define AWGN

Ans: AWGN (Additive White Gaussian Noise) is a noise model in which the noise is additive, has a constant power spectral density over all frequencies, and follows a Gaussian amplitude distribution.

### Q22. Unit of noise spectral density

Ans: The unit of noise spectral density is W/Hz.

### Q23. Why is SNR expressed in dB?

Ans: SNR is expressed in decibels to conveniently represent very large ratios and to simplify system analysis involving multiplication into addition.

### Q24. State Friis equation

Ans: The Friis equation for noise figure of cascaded systems is:

F total = F1 + [(F2−1)/G1] + [(F3−1)/G1G2] +…………..

Where F1,F2,F3,……..=Noise factors of individual stages(linear)

G1,G2,…………=Power gains of stages(Linear)

### Q25. What is receiver sensitivity?

Ans: Receiver sensitivity is the minimum input signal power required at the receiver to achieve a specified output SNR.

### Q26. Why does Noise Figure not depend on input signal power?

Ans: Because it measures relative SNR degradation, independent of absolute signal level.

### Q27. If gain increases, does Noise Figure always decrease?

Ans: No. Noise Figure depends mainly on the first stage noise, not only gain.

### Q28. Why do we assume thermal noise as white?

Ans: Because its power spectral density is constant over communication bandwidths.

### Q29. What happens to SNR if bandwidth is doubled?

Ans: Noise power doubles, hence SNR reduces.

### Q30. Why is dB scale preferred in receiver analysis?

Ans: It simplifies cascaded system calculations and large ratios.

### Q31. Why Noise Figure is important?

Ans: It measures receiver performance.

### Q32. Why is white noise modeled as Gaussian noise?

Ans: White noise is modeled as Gaussian noise because the noise in practical communication systems arises from a large number of independent random processes. According to the Central Limit Theorem, the resultant noise follows a Gaussian probability distribution.

### Q33. What is the significance of Noise Figure in receivers?

Ans: Noise Figure indicates how much a receiver degrades the Signal-to-Noise Ratio. A lower Noise Figure implies better receiver sensitivity and improved detection of weak signals.

### Q34.Why does the noise spectrum appear flat?

Ans: White noise has constant power spectral density, meaning equal power is distributed over all frequencies. Hence, its frequency spectrum appears flat.

### Q35. Difference between SNR and Noise Figure?

| Parameter | SNR | Noise Figure |
| --- | --- | --- |
| Definition | Ratio of signal power to noise power | Measure of SNR degradation |
| Location | Can be measured anywhere | Property of a system |
| Unit | dB | dB |
| Indicates | Signal quality | System noise performance |

Bottom of Form