PCM Matlab code

clc;

close all;

clear all;

amp=input('Enter Amplitude of the signal: ');

f=input('Enter Frequency of the signal: ');

fs=input('Enter sampling frequency : ');

n=input('Enter n value for n-bit PCM system :  ');

L=2^n; 				%Number of Quantization levels

t=0:0.0001:1;               	% n1 number of samples

t1=0:1/fs/f:1;

s=amp*sin(2*pi*f*t);  		%input signal

s1=amp*sin(2*pi*f*t1); 		%sampled signal

subplot(3,1,1);

plot(t,s);

title('Analog Signal');

ylabel('Amplitude--->');

xlabel('Time--->');

subplot(3,1,2);

stem(t1,s1);

grid on;  title('Sampled Sinal');

ylabel('Amplitude--->');

xlabel('Time--->');

%  Quantization Process

vmax=amp;

vmin=-vmax;

del=(vmax-vmin)/L;

part=vmin:del:vmax; 	% level are between vmin and vmax with difference of del

code=vmin-(del/2):del:vmax+(del/2);            % Contain Quantized values

[ind,q]=quantiz(s1,part,code);                  % Quantization process

% ind contain index number and q contain quantized  values

l1=length(ind);

l2=length(q);

for i=1:l1

if(ind(i)~=0)         % To make index as binary decimal so started from 0 to N

ind(i)=ind(i)-1;

end

i=i+1;

end

for i=1:l2

if(q(i)==vmin-(del/2))      % To make quantized values in between the levels

q(i)=vmin+(del/2);

end

end

subplot(3,1,3);

stem(t1,q);

grid on;                               % Display the Quantized values

title('Quantized Signal');

ylabel('Amplitude--->');

xlabel('Time--->');

%  Encoding Process

figure                                          % new figure window

code=de2bi(ind,'left-msb');                    % Convert the decimal to binary

k=1;

for i=1:l1

for j=1:n

coded(k)=code(i,j);             % convert code matrix to a coded row vector

j=j+1;

k=k+1;

end

i=i+1;

end

subplot(2,1,1); grid on;

stairs(coded);                                 % Display the encoded signal

axis([0 fs*n*f -2 3]);  title('Encoded Signal');

ylabel('Amplitude--->');

xlabel('Time--->');

grid on;